CONTROL OPTIONS FOR COAL-FIRED BOILERSa

Table 1.1-2.  NOx

Reduction	Commercial

NOx

Description of	Applicable Boiler	Potentialb	Availability/R & D

Control Technique	Technique	Designs	(%)	Status	Comments

Combustion Modifications

Load reduction	Reduction of coal Stokers	Minimal	Available	Applicable to stokers that can reduce load without and air	increasing excess air; may cause reduction in boiler  reduction varies with percent load

efficiency; NOx

reduction.

Operational	Rearrangement of Pulverized coal	10 - 20	Available	Must have sufficient operational flexibility to achieve modifications (BOOS,	air or fuel in the	boilers (some	 reduction potential without sacrificing boiler

NOx

LEA, BF, or	main combustion designs); Stokers	performance. combination)	zone	(LEA only)

Overfire Air	Injection of air	Pulverized coal	20 - 30	Available	Must have sufficient furnace height above top row of above main	boilers and stokers	burners in order to retrofit this technology to existing combustion zone	boilers.

Burners	New burner	Pulverized coal	35 - 55	Available	Available in new boiler designs and can be retrofit in

Low NOx

designs	boilers	existing boilers. controlling air-fuel mixing

LNB with OFA	Combination of	Pulverized coal	40 - 60	Available	Available in new boiler designs and can be retrofit in new burner	boilers	existing boilers with sufficient furnace height above designs and	top row of burners. injection of air above main combustion zone

Reburn	Injection of	Pulverized coal	50 - 60	Commercially	Reburn fuel can be natural gas, fuel oil, or pulverized reburn fuel and	boilers, cyclone	available but not	coal.  Must have sufficient furnace height to retrofit completion air	furnaces	widely demonstrated this technology to existing boilers. above main combustion zone