# A PRELIMINARY INVENTORY OF VEHICLE POLLUTANT EMISSIONS IN THE CITY OF CAPE TOWN

## Juliet J. Mert &  Eugene K. Cairncross

#### Department of Physical Science, Peninsula Technikon, Box 1906, Bellville 7535, South Africa, cairncrosse@pentech.ac.za

## ABSTRACT

The preparation of a comprehensive database of pollutant emissions in the form of an emission inventory is one of the basic tools for assessing urban air quality. The need to integrate emission data from a large number of sources, and the complexity of the relationship between the activity of generating pollutant emissions, and the emission rate, necessitates the use of computer modeling tools. The IPIECA Toolkit model is a Microsoft® ExcelTM based tool for estimating emissions from both stationary and mobile sources. Domestic, commercial, industrial and mobile (vehicular) sources may be integrated to yield outputs. The Toolkit was successfully applied to the Cape Metropolitan Area. The Base Case for the area generated emission estimates for hydrocarbons, particulate matter, carbon monoxide, lead and oxides of nitrogen. The modeled outputs provided a reasonable match for fuel consumption in the study area, and successfully modeled proposed interventions to eliminate leaded petrol, reduce sulphur in diesel and introduce catalytic converters over the next five to ten years.

## KEYWORDS: Emission Inventory, Mobile Source Emissions, Urban Air Quality

## INTRODUCTION

Air pollution is a major problem in cities worldwide, affecting both developed and developing countries. The increasing amounts of potentially harmful gases and particulates being emitted are destructive to human health and the environment. The three major sources of air pollution that result from human activity are stationary sources (from factory stacks and waste heat boilers, etc.), mobile sources (from heavy- and light duty vehicles and motorcycles, etc.) and indoor sources (from paraffin lamps, gas cookers and domestic fires, etc.). These air pollutants are commonly classified into suspended particulate matter (dusts, fumes, mists and smokes), gaseous pollutants (gases and vapors) and odors. The list of pollutants, common to most cities, include carbon monoxide (CO), nitrogen dioxide (NO2), sulphur dioxide (SO2), ozone (O3) and particulate matter (PM), and are known as the classical pollutants. Other pollutants, typically vehicle emitted, include volatile organic compounds (VOCs) and lead (Pb).

The health impacts of these pollutants range from neurobehavioral effects, such as impaired coordination, driving inability and defective cognitive performance resulting from long-term exposure to CO, to asthmatic aggravation from prolonged exposure to NO2, SO2 and O3. Mortality and increases in rates of bronchitis and reduced lung function have been reported as a result of prolonged exposure to low concentrations of PM (less than 100g/m3) [WHO, 2000]. VOCs comprises of a list of hydrocarbons most of which either have carcinogenic or non-carcinogenic health end points. Toluene and the xylenes (ortho and para), for example, are non-carcinogens that affect the central nervous system (CNS) in humans. Toluene is also known to cause fetal developmental effects. Trichloroethylene is a carcinogen in addition to having effects on the liver and CNS. Benzene is another known human carcinogen, typically causing leukemia in exposed humans. Pb affects the CNS, blood pressure, kidneys and vitamin D metabolism in humans. Children are most affected by lead emissions resulting in slowed cognitive development and reduced growth.

The World Health Organization (WHO) has set up an Air Management Information System, AMIS (WHO 1997b) that is currently the main source of information on air pollution in developing countries (WHO, 2000). AMIS has reported that the annual mean concentrations of SO2, in the residential areas of most cities, has not exceeded the WHO guideline of 50g/m3 with the exception of 330 and 100g/m3 in Chongqing and Beijing, respectively, in 1994. In the case of total suspended particulates AMIS reports that most cities exceeds 100g/m3, with levels exceeding 300g/m3 in several cities of China and India. For most cities AMIS reports annual mean concentrations of NO2 below 40g/m3 (which is below the WHO guideline of 40 – 50g/m3). Mexico City and Cape Town are the exceptions with an annual average of 70g/m3 that was exceeded frequently in the 1990s.

In order to estimate the effects of air pollutant exposure on the population, ambient air quality monitoring and assessment is required. These assessments provide decision makers with the information they require to make informed decisions on managing and improving the environment. The three main air quality assessment tools include ambient monitoring, models and emission inventories.

Monitoring stations are set up to monitor a specified list of pollutants. The City of Cape Town, for example, monitors NO2, SO2, PM, O3 and Pb at nine monitoring sites. Limitations of monitoring include the fact that it cannot comprehensively quantify patterns of air pollution in both space and time. Furthermore, it cannot be used to predict future levels of air pollution and are, generally, very expensive. Monitoring does, however, provide an incomplete picture of current environmental quality and thus, needs to be used in conjunction with other assessment tools (such as modeling and emission inventories) to provide a complete picture of population exposure.

Modeling can provide a powerful tool for interpolation, prediction and optimization of control strategies, but they depend on the availability of reliable emission data and should, thus, not be relied upon completely. A model typically requires a complete emission inventory (including emissions from point, area and mobile sources) for the study area. It is important to investigate whether the model used is appropriate to local conditions, sources and topography.

An emission inventory (EI) is a general database of all significant pollutants released from all sources (stationary, mobile and area sources) to the atmosphere, including the corresponding emission rates and source identification data. Despite being an air quality assessment tool itself; an EI is a key component of modeling. The EI should cover an appropriate range of potentially harmful (directly or indirectly) substances, and should include information on sources and release rates in sufficient detail to enable intervention to reduce actual or potential impacts. The inventory should be sufficiently accurate to enable source apportionment of specific pollutants, essential for an Air Quality Management intervention. Of course an assessment of exposure (for example using air quality modeling) and of health and environmental impacts requires additional source characterisation data that may not be available in the EI database. In practice, data characterizing stationary sources (physical coordinates, stack heights, etc.) frequently resides in a different database to that of a general emission inventory. In addition, the stationary source emission inventory is frequently separate from the mobile sources emission inventory, as is the case with the United States’ Toxic Release Inventory and the North American (essentially the United States and Canada) Pollutant Release and Transfer Register. The balance between the value of developing a single database including all source and emission rate data relevant to an Air Quality Management system and the complexity of such a system is a matter of debate.

It is common cause that South Africa is in urgent need of the development and implementation of an Air Quality Management system (AQMS). The development of a Dynamic Air Pollution Prediction System (DAPPS) is a multi-centre, three year project with the objective of incorporating all pollutant emission sources (stationary and mobile), atmospheric chemistry and pollutant dispersion modelling, to predict a health based air pollution index for the Cape Town area. DAPPS proposes to develop an information system to assist stakeholders in managing air quality in the City of Cape Town.

One of the requirements of DAPPS is a mobile source emission inventory. The IPIECA Toolkit model has been used to generate the mobile source emission inventory for the Cape Town area. The Toolkit has the ability to model the emissions from both mobile (vehicular) and stationary sources, and hence provides a means of apportioning the relative contributions to the total of these two broad categories. For the mobile source emissions, the model estimates emission rates based on activity rates such as vehicle kilometers traveled and fuel consumption rates against the predicted annual human population and vehicle growth rates. Additionally, time dependent regulatory interventions may be used to predict the increase (or decrease) in emissions. The value of this ability is that the model enables the evaluation of different scenarios or intervention strategies, which is an important consideration in the development of air quality management plans.

Individual vehicle emission rates (both tailpipe and fugitive emissions, g pollutant per km travelled) are a function of interrelated factors that include fuel composition, vehicle type and size (mass), driving speed, engine design, state of vehicle maintenance, altitude and emission control systems. Total vehicle emissions in a given area are the sum of emissions from each vehicle on the roads in the study area during the time period of interest. It would be impractical to measure a set of emission factors for each vehicle. Thus, the methodology used (in North America, Europe and other industrialized regions) to estimate total emissions categorizes the vehicle population by fuel type and size classes with statistical sampling of each category and measurements of emission factors of the sample under standardised driving cycles,. The considerable investment (in time and money) required to generate these large empirical databases of emission factors is not available in South Africa. It is therefore necessary to use the available databases as initial or default values. The default values may be modified using applicable local data. Historically, the South African vehicle population fleet more closely matches that of Europe than North America; thus the European default values embedded in the IPIECA Toolkit are appropriate.

In view of our backlog of work in the development and implementation of an AQMS, and the demands on resources - financial, human and time - required to put a system in place as quickly as possible, the practicality of customizing available software (such as the Toolkit) is an important matter. Of course the accuracy (validity) and precision of model outputs is dependent on the quality of data inputs, and the validity of the assumptions and structure of the model. A full implementation of the IPIECA emission model (or any other model) requires statistically robust procedures for verifying the input data as well as verifying that the calculation methods and emission factors embedded in the model are applicable to the local situation. This may be ascertained through the independent measurement of at least a few values, comparison against the limited locally generated data, and vehicle emission studies using ambient measurements in a road tunnel such as the Huguenot Tunnel. The main objective in this study would be to verify the mobile source emission factors embedded in the IPIECA Toolkit, locally, using vehicle emissions (mobile sources) from the Huguenot tunnel. From the list of vehicle pollutants emitted (VOCs, SOx’s, CO, PM, NOx’s and Pb), this study will focus on VOCs, CO and PM. The Huguenot tunnel represents a real-world lab (i.e. a closed environment) within which the environmental factors (such as temperature and humidity) can be assumed fairly constant, and has therefore been chosen as the study area.

Finally, we need to assess and balance the accuracy and precision achievable with a particular methodology against the time and cost incurred in improving that precision.

## METHODOLOGY

The IPIECA Toolkit was used to construct a base case for mobile emissions in the Cape Metropolitan Area. The base year was chosen as 2001 due to the lack of some post 2001 data. The development of the model involved the identification and gathering of input data, and thus the development of the mobile source emission inventory. In cases where data was not available, reasonable assumptions were made or the default values were used. For example, the model requires average speeds for urban, rural and highway driving and annual average mileage for each vehicle class, for each driving mode – urban, rural and highway – but these figures are not readily available, thus the default values were used and adjusted.

The IPIECA Toolkit Model uses standard procedures for estimating mobile source emissions (IPIECA Toolkit Reports, European Union Emission Inventory Guidebook). The procedure for estimating mobile source emissions is summarized in the flowchart, Figure 3.

Figure 3: Procedure for estimating mobile source emissions

CHARACTERISATION OF					 FACTORS AFFECTING

MOBILE SOURCES							EMISSIONS

INPUT DATA:			  	OUTPUT:			INPUT DATA:

Small

Medium	Petrol

Large

Passenger         Cars

Small

Medium	Diesel

Large

LDV		Commercial

HDV		Vehicles

Buses

2T		Motorcycles

4T

…………………………………….

Vehicle Lifetime Function, Scrappage Rate

Fuel Composition(s)

Mode of Use (Urban, Rural, Highway)

Annual Mileage

Travel Speed

Fuel Consumption

Emission Factors: Variation with speed, age degradation factors, engine design

MOBILE SOURCE EMISSONS PROFILE

(SO2, Pb, HC, NOx, CO)

Petrol and Diesel Consumption

(Fuel Balance)

Total Emissions

Total Emissions

per Vehicle Class

Vehicle population by size and type

The car population is divided into three size classes:

- small cars have engine displacement below 1400 cc

- medium cars have engine displacement between 1400 cc and 2000 cc

- large cars have engine displacement above 2000 cc.

Survival Rates

The model requires historical car population data and the survival rate (or scrapping rate), modeled using the two-parameter Weibull distribution function in order to predict the future car population. The Weibull distribution is commonly used to model the failure of mechanical components as a function of time. It gives the probability of survival (or failure) as a function of time, with failure rates increasing with time [Lapin]. The two parameters were determined using the weibull distribution probability plotting method. The shape (β) and time (η) parameters were found to be 3.61 and 18 respectively. For the purpose of the development of the CMA Base Case, national survival (scrapping) rates were applied to the Western Cape car population data. Figure 4 shows the historical data, and the cumulative IPIECA Toolkit generated Weibull distribution curve.

Figure 4: Actual National Survival Rates vs Weibull Distribution

The actual average scrapping rate curve is influenced by factors such as the year-to-year variation in new vehicle sales, and does not follow a smooth curve. Thus the curve was fit using more recent data, from 1980 to 2000. The model predicts annual vehicle sales based on the population figures and the Weibull distribution. The predicted sales may be compared against the actual sales, Table 6 in the results section.

Fuel Composition

###### Table 2: Fuel Composition Used*

|  | Grade 1 | Grade 2 |
| --- | --- | --- |
|  | Specification | Specification |
| %Evaporated at 100C | 63% | 49% |
| %Evaporated at 150C | 145% | 135% |
| Aromatic content (vol%) | 27.7% | 0.8% |
| Sulphur content (weight ppm) | 900 | 300 |
| Benzene (vol%) | 1.1% | 40.1% |
| Summer RVP @ 37.8oC (kPa) | 70 | 70 |
| Density @ 20oC (kg/m3) | 730 | 740 |
| RON | 97 | 95 |
| MON | 87 | 85 |
| LEAD g/litre @15oC | 0.34 | 0.0010 |

*Fuel composition data provided by Caltex

The toolkit makes provision for 5 gasoline and diesel scenarios. These scenarios were manipulated to represent the different phases of gasoline and diesel composition pre- and post 2001. Thus, the introduction of catalysts on all new vehicles in 2008 and the phase out of lead in 2006 is accounted for. The reduction of sulphur in diesel is also depicted, decreasing from 5500ppm to 3000ppm in 2002 and a further reduction to 500ppm in 2006. The grade 1 (97 leaded) and grade 2 (95 unleaded) split is estimated at 100:0% pre 1986, 75:25% post 1986 and 0:100% post 2006 (SAPIA).

Factors for the calculation of Mobile (Vehicle) Emissions

Default values for annual kilometers traveled were generally used, that is, 15 000 to 20 000kms per annum. Urban, rural and highway speeds of 25, 80 and 90 km/h were assumed. The fuel consumption figures are given in Table 3. Figure 5 was generated from the mobile source emission factors embedded in the Toolkit. The figure depicts the behavior of various emissions without controls at different traveling speeds.

##### Table 3: Assumed Fuel Consumption for Petrol Driven Cars

| ## Mode | ##### Vehicle Size |  |  |
| --- | --- | --- | --- |
|  | Large | Medium | Small |
| Urban | 21.0 | 12.2 | 10.0 |
| Rural | 13.0 | 10.5 | 7.8 |
| Highway | 12.6 | 10.0 | 7.3 |

##### Figure 5: Emission Rates vs Car Speed (without emission controls)*

*Only applicable for the speed range between 5 and 90km/hr

The questions of validation (assessing the model output for internal consistency) and verification of the input data, emission factors and, where possible, the model outputs against locally measured data are discussed against a background of limited and partial data sources.

## The study area

The study area was defined as the boundaries of the Cape Metropolitan Area (CMA), approximately 2487km2:

Figure 1: Map of the Cape Metropolitan Area (CMA)

This definition of the study area (Cape Metropolitan Council) is convenient from a data gathering point of view, and is a reasonable approximation to an ‘air shed’, at least from the perspective of the location of significant emission sources.

Human Population Data

This study used the Cape Metropolitan Council’s State of the Environment Report [Year Two] population estimate of 3.11 million in 2001, and a growth rate of 2.0% [Bekker and Van Zyl, 1998], as a basis for extrapolating population growth to the year 2025. The model uses the human population projection and the number of dwellings in the area to estimate future energy demand and fuel consumption as a basis for predicting emissions from domestic sources. A constant ratio of four persons per household, based on the 1996 CMA statistics [SoE, 1999], was used.

## Figure 2: Projected Growth Rates of Human Population and Number of Dwellings

Mobile Source Emissions

The model requires total year-by-year vehicle population figures, and a percentage allocation by size category. It is possible to vary the vehicle size split on a yearly basis, but constant figures for the period 1970 to 2000 (15%, 75%, 10% for small, medium and large cars respectively) were used. In 1998, the Cape Metropolitan Area had about 170 vehicles per 1000 people and about 3 million people (State of the Environment Report), giving a vehicle population of about 510 000. Data drawn and estimated from the Annual Naamsa (National Association of Automobile Manufacturers of South Africa) Report [2001/2002] gives the following vehicle population figures for 2001 (See Table 1).

##### Table 1: Vehicle population by type, Western Cape (WC)

|  | Percentage by Type (All Vehicles) |  |  |
| --- | --- | --- | --- |
|  | Cars | MCa | CommVeh'sb |
| Western Cape as % of National | 18% | 20% | 14% |
| WC Population as at 2001 | 805456 | 31127 | 225713 |
| Assume CMA is 70% of WC | 563819 | 21789 | 157999 |

a MC: Motor Cycle; b CommVeh’s: Commercial Vehicles

For the purpose of developing this Base Case Model, the vehicle population (percentage) of the CMA was calculated from 2001 values, from the SoE report, for the CMA vehicle population (825000) and the 2001 Annual Naamsa report for the Western Cape total (1062296), and was found to be approximately 80% of the Western Cape figures. Vehicle projections, however, agreed better with 70%. The CMA distribution of vehicles between the various types was assumed to be the same as the provincial figures. The Table 1 figures were used in the Base Case model.

## RESULTS

The Base Case was run to estimate mobile source emissions that is, without stationary source (domestic and industrial) input data. A basic check of the overall validity of the input data and the model structure was done by comparing the predicted petrol and diesel consumption figures against actual sales (a fuel balance), and the predicted new vehicle sales against actual figures provide. As was the case for the input data, specific data for the CMA were not available at the time of writing. Thus CMA sales were estimated at 70% of Western Cape sales, the same factor used for the input data.

##### Table 6: Total Petrol and Diesel Consumption, Modelled vs Actual

|  | Total Consumption, Petrol (kT) |  | Total Consumption, diesel (kT) |  |
| --- | --- | --- | --- | --- |
|  | Modelled | Estimated actuala | Modelled | Estimated actuala |
| 1995 | 870 | 953 | 595 | 510 |
| 1996 | 892 | 992 | 627 | 541 |
| 1997 | 929 | 1012 | 655 | 551 |
| 1998 | 972 | 1022 | 686 | 559 |
| 1999 | 972 | 1020 | 687 | 563 |
| 2000 | 972 | 976 | 689 | 587 |
| 2001 | 974 | 971 | 691 | 609 |

a: Estimated, based on South African Petroleum Industry Association (SAPIA) data.

##### Table 7: Car Population and New Car Sales, Predicted vs Estimated Actual [millions]

|  | Car Population |  | New Car sales |  |
| --- | --- | --- | --- | --- |
| Year | Predicted | Estimated actual | Predicted | Estimated actual |
| 1995 | 0.511 | 0.517 | 0.052 | 0.029 |
| 1996 | 0.538 | 0.545 | 0.061 | 0.031 |
| 1997 | 0.562 | 0.568 | 0.058 | 0.030 |
| 1998 | 0.588 | 0.510a | 0.063 | 0.025 |
| 1999 | 0.589 | 0.589 | 0.038 | 0.023 |
| 2000 | 0.590 | 0.597 | 0.039 | 0.028 |
| 2001 | 0.592 | 0.599 | 0.041 | 0.029 |

a: State of the Environment Report, 1998 estimate.

Actual sales of fuel and vehicles in fact do not follow a smooth exponential growth curve. The model parameters may be tuned to give a good fit for a particular year (the current model has only been roughly tuned), but some divergence is to be expected over a multi-year period.

Examining predicted pollutant emissions outputs best demonstrates the value of modeling, as against an inventory that is valid only in a particular year. The following graphs (Figures 6 to 8) reflect a scenario predicting the introduction of 500ppm diesel sulphur, the introduction of catalytic converters on all new vehicles by 2008 and the planned phase-out of leaded petrol by 2006. Pollutant emissions are given as thousand tons per annum, kT/a.

Figure 6: CMA Mobile Source Emissions of Particulate Matter

## Figure 7: CMA Mobile Source Emissions of Hydrocarbons

Figure 8: CMA Mobile Source Emissions of Lead

## DISCUSSION

The Base Case model is complete; some fine-tuning may still be done to ensure an even closer fit. The estimated vehicle population data for the CMA has been verified by cross-referencing the data against SoE reports and the annual Naamsa report for 2001/2002. Conducting a statistically based survey of the vehicle population, although resource intensive, would yield useful results for annual kilometers traveled, the split between urban, rural and highway driving etc. Actual fuel consumption data specific to the study region as against figures factored from the Western Cape data should be used. The results presented in this paper, serves to present the total annual emissions for the CMA. Furthermore, it illustrates some of the IPIECA Toolkit’s capabilities.

Some inflexibility in the model’s present structure is apparent. Emissions from households appear to be modeled as if the fuels used (electricity, wood etc.) applied uniformly to all households. A facility to segment households into different categories based on the predominant fuel source may be more useful for constructing different scenarios. Local conditions also require specific modeling of emissions from the microbus taxi fleet. The default factors (for the size class) for vehicle kilometers traveled, modes of travel and average speeds are unlikely to apply to the taxi fleet. It would be necessary to establish these factors through appropriate research, and the construction of a separate Base Case model.

The model estimate for total fuel consumption (Energy Balance Output) may be compared against actual fuel sales, but good agreement does not necessarily guarantee a correct model structure because different combinations of average speed, modes of travel (urban, rural and highway) and vehicle kilometers traveled may yield the same total fuel consumption. The correctness of the structure of the model is of particular importance when analyzing different intervention scenarios, and the associated costs. In these cases, the sensitivity to possible errors in the input data should be ascertained.

The applicability of the model’s default emission factors may be ascertained through independent measurement of at least a few values, comparison against the limited locally generated data, and vehicle emission studies using ambient measurements in a road tunnel such as the Huguenot Tunnel.

In the Durban emission inventory emission factors, determined for the South African vehicle fleet, were used to estimate emissions from vehicular traffic. Similarly, in the Brown Haze Study [1997] emissions were determined by multiplying fuel consumption by emission factors. In both these cases fuel composition and fuel scenarios have not been considered. Thus, any change in these two parameters would not be reflected in the inventory.

Table 8: Comparison of Brown Haze Report emission factors to estimated emission factors estimated from this modeling excercise

| Study | Petrol |  | Diesel |  |
| --- | --- | --- | --- | --- |
|  | Hydrocarbons | Particulates | Hydrocarbons | Particulates |
| Brown Haze | 36 | 0.16 | 5.0 | 6.6 |
| IPIECA* | 14.4 | 0.02 | 9.2 | 6.5 |

*These emission factors were obtained by dividing the emissions for the base year (2001) by the fuel consumption for that year.

Except for particulates, the overall emission factors estimated using the IPIECA Toolkit differ substantially from those used in the Brown Haze Study. The agreement for particulates is co-incidental as the sulphur content of diesel (a major factor in particulate formation) has dropped from 5 500ppm to 3 000ppm in the interim.

A detailed speciation of the hydrocarbon emissions is required in order to fully characterize the health risk associated with this pollutant stream.

## CONCLUSIONS

The IPIECA Toolkit model provides a promising tool for the estimation of a regional emission inventory. However, the input data needs are considerable, and this aspect of the preparation of an emission inventory should not be underestimated.

A major strength of using a model capable of accounting for changes in fuel specification and vehicle emission control technology, as well as traffic patterns, is that the benefit of various intervention strategies may be explored using the model.

Further validation of the IPIECA model’s emission factors is planned in the Huguenot Tunnel Study Project.

The trends in figures 6 to 7 indicate that the Toolkit has been successful in generating a mobile source emission inventory that reflects the phase-out of leaded petrol (2006), further reduction of the diesel sulphur content and the introduction of catalytic converters (2008).

## Acknowledgements

This research was partly funded through the DAPPS project, funded by the Innovation Fund, administered by the National Research Foundation. The IPIECA (International Petroleum Industry Environmental Conservation Association) software was supplied free of charge through IPIECA/ SAPIA (South African Petroleum Industry Association). The Peninsula Technikon provided financial support to a student during the initial data-gathering phase of the work.

## References

Bekker, S. and Van Zyl, B. (1998). Baseline study on Urbanization in the Cape Metropolitan Area.

Cape Metropolitan Council, State of the Environment Report (SoE) for the Cape Metropolitan Area Year Two 1999, 1 September 2000, www.cmc.gov.za,

Coe, S. Caltex. Email Communication. May/June 2003.

European Union Emission Inventory Guidebook, Road Transport, Report rt070100 (September 1999)

IPIECA (2001). Urban Air Quality Management Volume 2 – The preparation and application of pollutant emission inventories: exploring the capacity of the IPIECA Toolkit. IPIECA, London.

Lapin L. L. (1990). Probability and Statistics for Modern Engineering, PWS KENT, (2nd edn.).

National Association of Automobile of Manufacturers of South Africa (Naamsa) Annual Report, 2001/2002.

Wicking-Baird, M.C., de Villiers, M.G., Dutkiewicz, R.K. 1997. Cape Town Brown Haze Study. Report No. GEN 182.

SAPIA: South African Petroleum Industries Association

State of the Environment Report (SoE) for the Cape Metropolitan Area Year Two 1999, 1 September 2000, www.cmc.gov.za, accessed July 2002.

WHO (1997). Air Management Information System AMIS 1.0. WHO, Geneva.

http://ceroi.net/reports/durban/issues/air/screen1.htm

International Petroleum Institute Environmental Conservation (IPIECA) Toolkit

Ref to Mobile5 and supporting docs

Ref to Eu Auto-oil docs