Table 9.1.  POSTCOMBUSTION SO2 CONTROLS FOR COAL COMBUSTION SOURCES

| Control Technology | Process | Typical Control Efficiencies | Remarks |
| --- | --- | --- | --- |
| Wet scrubber | Lime/limestone | 80 - 95+% | Applicable to high sulfur fuels, wet sludge product |
|  | Sodium carbonate | 80 - 98% | 5-430 million Btu/hr typical application range, high reagent costs |
|  | Magnesium oxide/ hydroxide | 80 - 95+% | Can be regenerated |
|  | Dual alkali | 90 - 96% | Uses lime to regenerate sodium-based scrubbing liquor |
| Spray drying |  |  |  |

Calcium hydroxide	70 - 90%	Applicable to low and   slurry, vaporizes in	medium sulfur fuels,   spray vessel	produces dry product

Furnace injection	Dry calcium	25 - 50%	Commercialized in Europe,   carbonate/hydrate	several U. S. demonstration   injection in upper	projects are completed   furnace cavity

Several research and Duct injection	Dry sorbent injection	25 - 50+%	development, and   into duct, sometimes	demonstration projects   combined with water	underway, not yet   spray	commercially available in the United States.

Table 1.1-3.  EMISSION FACTORS FOR SOx, NOx, AND CO

FROM BITUMINOUS AND SUBBITUMINOUS COAL COMBUSTIONa

b	c	COd,e

SOx	NOx

Emission	EMISSION	Emission	EMISSION	Emission	EMISSION

Factor	FACTOR	Factor	FACTOR	Factor	FACTOR Firing Configuration	SCC	(lb/ton)	RATING	(lb/ton)	RATING	(lb/ton)	RATING

PC, dry bottom,	1-01-002-02	38S	A	22	A	0.5	A wall-firedf, bituminous 	1-02-002-02 Pre-NSPSg	1-03-002-06

PC, dry bottom,	1-01-002-02	38S	A	11	A	0.5	A wall-firedf, bituminous 	1-02-002-02 Pre-NSPSg 	1-03-002-06

with low-NOx

burner

PC, dry bottom,	1-01-002-02	38S	A	12	A	0.5	A wall-firedf, bituminous 	1-02-002-02 NSPSg	1-03-002-06

PC, dry bottom,	1-01-002-22	35S	A	12	C	0.5	A wall-firedf, sub-bituminous 	1-02-002-22 Pre-NSPSg	1-03-002-22

PC, dry bottom, wall firedf,	1-01-002-22	35S	A	7.4	A	0.5	A sub-bituminous NSPSg	1-02-002-22 1-03-002-22

PC, dry bottom, cell burnerh	1-01-002-15	38S	A	31	A	0.5	A fired, bituminous

PC, dry bottom, cell burner	1-01-002-35	35S	A	14	E	0.5	A fired, sub-bituminous

Compilation of  Air Pollutant Emission Factors (AP-42), US Environmental Protection Agency (2003), www.epa.gov