#### 3.2 Area Noise Levels

An area or location is considered a noise hazard if sound levels are regularly at, or above, 85 dBA.

### TABLE 1: EQUIVALENT NOISE EXPOSURES

| Duration per Day (hours) | Maximum Permissible Exposure Durations for Noise without Hearing Protection (Continuous or Intermittent Noise) * |
| --- | --- |
| 16 | 82 |
| 8 | 85 |
| 4 | 88 |
| 2 | 91 |
| 1 | 94 |
| 1/2 | 97 |
| 1/4 | 100 |
| 1/8 | 103 |
| 0 | 106 |

* Based on a 3 dB exchange rate

### TABLE 2: IMPACT NOISE

| Sound Level (dB) | Maximum Number of Impacts/ 8 hour Day |
| --- | --- |
| >140 | 0 |
| 140 | 100 |
| 130 | 1,000 |
| 120 | 10,000 |